A Universal DFT Verification Environment: Filling the Gap between Function Simulation and ATE Test

Rui Huang

No.2, Science Institute South Rd.

Haidian District, Beijing, China 100190

Abstract-The DFT (Design For Testability) design has become more and more complex accompanying the increasing scale of SoC (System on Chip).  How to verify DFT logic completely in simulation and how to supply test patterns with high coverage to ATE (Automatic Test Equipment) test are important for post-silicon debug and yield increase.  While verification methodology is evolving, innovating and entering the UVM (Universal Verification Methodology) era, DFT verification needs to keep pace to leverage the advantages of UVM, and thereby to increase test reusability, extendibility and function coverage, etc.  This paper presents a general UVM-based DFT verification environment, which can be used from modular DFT verification to SoC DFT verification, and it can generate functionally equivalent STIL (Standard Test Interface Language) test patterns for ATE test during SoC simulation.  This paper also presents a method to model hierarchically networked DFT TDR (Test Data Register) at RAL (Register Abstract Level) in the UVM environment to allow test writers focus on test sequences without taking care of the details in TDR read and write operations.

# I.   Introduction

In DFT (Design For Testability) domain, the test patterns running on an ATE (Automatic Test Equipment) can be categorized into two types: scan related and non-scan related.  The former can be generated using ATPG (Automatic Test Pattern Generation) tools, while the latter cannot.  Like other function tests, these non-scan DFT function tests are normally created by design verification engineers using languages such as System Verilog or C++.  However, ATEs need test patterns described by STIL (Standard Test Interface Language) or other test languages.

To fill the gap, there is usually a dedicated team to transfer function simulation to ATE test environment, or alternatively in-house automation flows are developed to enforce complex rules on test writing and register specification documentation, which are specific for a given environment and difficult to migrate.

This paper provides a universal and more efficient solution by introducing a UVM (Universal Verification Methodology) based DFT verification environment that naturally generates test patterns in STIL format during simulation and can be plugged into any UVM-based environment.  This method applies to other formats that ATEs need as well.

For ultra-large-scale SoC (System on Chip), IEEE 1149 protocol alone cannot satisfy DFT design requirements, so IEEE 1687 and 1500 protocols are usually adopted to enable modular and hierarchical DFT test access, leading to challenges when writing test sequences at RAL (Register Abstract Level), as different protocol TDRs (Test Data Register) are hierarchically located in a network connected via IEEE 1687.  To access a TDR, one or more levels 1687 SIBs (Segment Insertion Bit) have to be set and the length of DR (Data Register) chain varies with SIB values. The author also comes up with a general way to model hierarchically networked DFT TDR (Test Data Register) at RAL.

## Structure of This Paper

This paper is divided into four parts.  The first part is about how to build a UVM-based DFT verification environment that can generate STIL test patterns naturally.  Then the second part will focus on the method of lifting DFT TDR to RAL.  The third part answers how to verify that the generated STIL pattern works.  The fourth part is the result discussion and conclusions.

In both of the first and second parts, the method we developed will be elaborated as follows: first, a general overview will be provided, and then the detailed implementation will be explained with reference to an example.

# II.   UVM-Based DFT Verification Environment

## Idea Overview

The STIL test pattern describes test stimulus using vectors which specify pad drive and measurement information (called STIL information hereinafter) in a time period.

A UVM test usually contains one or several sequences, which are finally broken down into streams of UVM sequence items (a.k.a transactions) and passed to UVM drivers.  UVM drivers are normally used for drive and sample pads of DUT, meaning that they also contain the STIL information passing through.  In fact, as to be demonstrated in this paper, the UVM drivers are the best supplier of STIL information.

With the precondition that any pad drive and sample are controlled by a UVM driver, which enforces no direct pad connection in the testbench (except for clock pads), simply by collecting all STIL information from the drivers and then writing them out according to the time stamp of STIL information, we can obtain complete test vectors of a certain UVM test when the simulation finishes.

Thus, we can divide the pads of a SoC into the following types for DFT functional simulation:

- IEEE 1149.1 compliance on-chip TAP (Test Access Port).  Hereinafter, it is simply called JTAG (Joint Test Action Group) interface as shown in Table I, which is the most important interface for DFT design.  Please note that in Table I, read_not_write signal is not defined in IEEE 1149.1, as it is an internal signal only used in this environment, for more description please refer to Section C.5.

- Clock pads, which are clocks that need to toggle in DFT functional simulation.  See Section D for more description.

- Reset pads.  All the reset related pads are categorized into this type.

- Other pads.  Except for type 1) to 3) abovementioned, the remaining pads are categorized into this type.  See Section E for more description.

In Fig. 1, jtag_driver, clock_driver, reset_driver, and pad_driver correspond to the above four pad types, respectively.  The STIL_generator collects STIL information from these drivers and writes them to a STIL pattern file.

## jtag_agent Implementation

In Fig. 2, jtag_agent is composed of jtag_sequencer, jtag_monitor, and jtag_driver, all of them configured through jtag_agent_configuration.

Figure 1. UVM-based DFT verification environment.

Table I

jtag Interface Definition

| JTAG Interface |  |
| --- | --- |
| Pad Direction | Pad Name |
| input | TCK |
| input | TMS |
| input | TRST_L |
| input | TDI |
| output | TDO |
| input | read_not_write |

Figure 2. jtag_agent block diagram.

### jtag_agent_configuration Class

Fig. 3 shows properties and a key method (pad_info_init ()) of jtag_agent_configuration class.

Figure 3. jtag_configuration properties and pad_info_init () method

jtag_vi is virtual JTAG interface.

reg_block is TDR register model in RAL.

gen_stil_file is configuration knob to turn on STIL pattern generation during simulation.

stil_file_name is desired STIL pattern file name.

pad_name array stores the JTAG interface package name of a SoC and the pad_dir array indicates pads direction.  STIL pattern files need this information to describe stimuli vectors.

A DFT function test must call pad_info_init () method to initialize pad_name and pad_dir arrays before UVM main_phase objection and store it in a configuration database for drivers and STIL_generator fetch.

### jtag_transaction Class

Fig. 4 shows properties of jtag_transaction class.

o_ir is a dynamic array to store instruction operation code (a.k.a OPCODE) sending to DUT’s IEEE 1149.1 FSM (Finite State Machine) IR (Instruction Register) and o_ir_length is its size.

o_dr is a dynamic array to store data sending to DUT’s IEEE 1149.1 FSM DR (Data Register) and o_dr_length is its size.

tdo_dr_queue, tdo_ir_queue, tdi_dr_queue, and tdi_ir_queue store data during shift IR or DR state monitored by jtag_monitor.

chk_ir_tdo and chk_dr_tdo are flags to indicate jtag_driver whether to check TDO cycle-by-cycle during shift IR or DR state.

exp_tdo_dr_queue is golden data expecting DUT TDO output during shift DR state, which is used by jtag_driver to check TDO data on the fly.

exp_tdo_dr_mask_queue indicates which bit in exp_tdo_dr_queue need not to check.

exp_tdo_ir_queue is golden data expecting DUT TDO output during shift IR state, which is used by jtag_driver to check TDO data on the fly.

read_not_write is a flag indicating jtag_monitor whether it is a read or write operation for current transaction. Please see Section C.5 for more details.

### JTAG Interface Connection in Testbench

This paper categorizes pads of a SoC into four types, which are driven by different drivers, so the JTAG interface shown in Table I is driven by clock_driver, reset_driver, and jtag_driver as shown in Fig. 5.

Fig. 6 is jtag_if definition that does not contain all signals shown in Table I because of the categorization of pads. The rest signals’ definition are defined in clock_if and reset_if interfaces.

### jtag_driver Class

IEEE 1149.1 protocol is implemented in jtag_driver, which fetches every jtag_transaction sequence item from jtag_sequencer, drives the JTAG interface’s TDI and TMS, and samples TDO if chk_ir_tdo or chk_dr_tdo flag is on. exp_tdo_dr_queue and exp_tdo_ir_queue store the expected golden value, which also will be used as the golden measure information for TDO in the generated STIL pattern.

If the gen_stil_file knob is on, jtag_driver not only needs to drive and sample pads – it also converts such information to STIL information (handled by call_stil_gen () method), and then sends it to STIL_generator through an analysis port, which is an object of uvm_analysis_port class specialized with stil_info_transaction type.

Figure 4. jtag_transaction properties definition.

Figure 5. JTAG interface toplevel connection.

Figure 6. Signals defined in jtag_if  interface.

In Fig. 2, let us suppose jtag_driver’s FSM is in shift DR state and it is going to shift three bits 101 to DUT and sample TDO data during shift operation. The golden TDO data is three bits 110.

At TCK negative edge a, jtag_driver keeps TSM low to let DUT’s FSM stay in shift DR state and drives TDI high to send out the first bit out. call_stil_gen () method converts this information as shown in line 1.

At TCK positive edge b, jtag_driver samples TDO and compares it with the golden value, which is one bit 1. call_stil_gen () method converts this information as shown in line 2.

At TCK negative edge c, jtag_driver keeps TSM low to let DUT’s FSM stay in shift DR state and drives TDI  low to send out the second bit out. call_stil_gen () method converts this information as shown in line 3.

At TCK positive edge d, jtag_driver samples TDO and compares it with the golden value, which is one bit 1. call_stil_gen () method converts this information as shown in line 4.

At TCK negative edge e, jtag_driver drives TSM low to let DUT’s FSM go to exit1 DR state and drives TDI high to send out the last bit out. call_stil_gen () method converts this information as shown in line 5.

At TCK positive edge f, jtag_driver samples TDO and compares it with the golden value, which is one bit 0. call_stil_gen () method converts this information as shown in line 6.

### jtag_monitor Class

There is a signal called read_not_write defined in JTAG interface shown in Table I, which is only used by jtag_monitor to indicate whether the current transaction is a write operation or read operation.

JTAG interface is a serial bus, while shifting TDI to a register, data stored in it is being shift out on TDO, so there is not a really so-called write or read operation.

Here, we define write operation and read operation in concept for RAL convenience.

Read operation: data being shifted in a register is the same as the data stored in it.

Write operation: data being shifted in a register is different with the data stored in it.

jtag_monitor monitors JTAG interface activity, sampling TDI or TDO according to read_not_write signal, composing jtag_transaction sequence items and then passing them to dft_tdr_laying as shown in the blue arrows of Fig. 1.

## Clock Pads Connection in Testbench

In STIL pattern file, the Timing block defines sets of “WaveformTables”.  Each WaveformTable defines the waveforms to be applied to each signal used in a vector [1].  Because DFT function tests only use JTAG interface to configure TDRs, we define one WaveformTable in the generated STIL pattern file and use TCK’s half period as WaveformTable’s Period.  For other clocks describe it as the same frequency as TCK in STIL pattern file and connect them with desired frequency form ATE during post-silicon test.  Therefore, clock_driver only need to drive TCK during simulation and other clocks are generated from testbench (This is the only exception that clock pads are allowed to drive from testbench in this environment).

Fig. 7 is an example of how to drive clocks in this DFT verification environment.

Figure 7. An example of clock pads connection in testbench.

Suppose a DUT has two PLL reference clocks and a bypass clock, which need active during simulation, named as PLL1_REF, PLL2_REF, and BYPASS_CLK as shown in Fig. 7.

clock_gen module in toplevel takes charge of these three clocks’ toggle.  TCK of JTAG interface is generated by clock_driver.

If gen_stil_file knob is on, clock_driver needs to pass TCK drive information to the call_stil_gen () method at the same time it drives TCK, and call_stil_gen () method uses the TCK drive information as all active clocks’ drive information and pass STIL information to STIL_generator as shown in Fig. 7 line1 to line4.

For an ATE test, PLL1_REF, PLL2_REF, and BYPASS_CLK toggle information in the STIL pattern can be regarded as a placeholder to make post silicon engineers aware that these three clocks are reference clocks, so that they will not use the toggle information described in STIL patterns to driver reference clocks, but use clocks supplied by ATE with desired frequency.

## pad_agent Implementation

Fig. 8 shows components in pad_agent and the execution flow in pad_driver, which fetches pad_rw_transaction from pad_sequencer.

The pad type 4) defined in Section B, can be subgrouped according to their function or interface protocol.  Take memory pads, GPIO pads, and scan control pads as examples, each of them could be put in a separate subgroup.

Fig. 9 is an example of subgrouping pads according to their interface protocol to define pad_if.

In Fig. 8, pad_init () method initializes all subgroups pads in turn at the beginning of run_phase task of pad_driver, and call_stil_gen () method converts this information to STIL information and writes to STIL_generator through an analysis port.

Fig. 10 displays all properties of pad_rw_transaction class.

grp_num is used to indicate pad_dirver which group of pads to drive.

in_data_queue stores data being driven by pad_driver.

out_data_queue stores data being sampled by pad_driver.

inout_data_queue stores data being driven or sampled by pad_driver.  An unknown bit in the queue indicates pad_driver the corresponding pad is in output mode and it will write the sampled pad value into the same location.

exp_out_data_queue and exp_inout_data_queue stores golden value to let pad_driver check on the fly and also the information for STIL pattern to measure pads value during a time period converting by call_stil_gen () method.

Please note these queue types should be logic instead of bit in order to store four state values.

Figure 8. pad_agent block diagram.

Figure 9. An example of defining pad_if in subgroups.

Figure 10. pad_rw_transaction properties definition

### pad_agent_configuration Class

Fig. 11 is an example of pad_agent_configuration class, which has two subgroups of pads.

A DFT test needs to initialize every group’s package name by calling the pad_info_init () method before the main phase objection and stores it in configuration database for pad_driver and STIL_generator fetch.

## reset_driver Class

Fig. 12 is an example of reset_driver that drives all resets signals defined in reset_if and call_stil_gen () method converts drive information to STIL information and writes to STIL_generator through an analysis port.

## STIL_generator Implementation

The STIL_generator, which extends from uvm_subscriber class specialized with stil_info_transaction type, has four analysis exports to connect with clock_driver, reset_driver, pad_driver, and jtag_driver’s analysis port separately.  Since the uvm_subscriber class has only one built-in analysis export, the uvm_analysis_imp_decl macro needs to be used to declare analysis imp export and its associated write () method for the remaining analysis export [2].

Figure 11. pad_agent_configuration properties definition example

Figure 12. An example of reset_driver.

stil_info_transaction is defined in Fig. 13.  stil_info is pads drive and measure information and comment info is the comment going to be printed out with the stil_info.

In Fig. 14, each driver’s analysis port has its corresponding write () method, a semaphore which has only one key and a group of Ping-Pong buffers which have two variables, called ping_data_rdy and pong_data_rdy, to indicate Ping-Pong buffer status.

The stil_info_transaction written through a driver’s analysis port is stored in a Ping-Pong buffer group, each buffer stores one stil_info_transaction.

The STIL_generator needs to collect all stil_info_transaction coming from the same simulation time slot, to concatenate stil_info of every stil_info_transaction, and to write them out as a single test vector.  To make sure STIL_generator does not miss any stil_info_transaction from the same time slot, it has to suspend the run_phase task in STIL_generator until all other run_phase tasks finish.  However, in UVM, because all uvm_component run_phase tasks are executed in parallel and the STIL_generator itself is an uvm_component, there is no easy way to schedule the simulation events in STIL_generator’s run_phase task to be executed after all other drivers’ run_phase task events finish.

To resolve this issue, a group of Ping-Pong buffers is introduced.  The write () method always writes the ping buffer first and then the pong buffer, so the ping data and pong data come at different simulation time slots.  Once a group of Ping-Pong buffers is full, which indicates the simulation has already moved forward, it will be the right time to collect all ping buffer data and write them out.

The run_phase task of STIL_generator, as shown in Fig. 14, always checks if there is at least a driver’s Ping-Pong buffer group is full.  If the result is true, it will query each key of the semaphore belonging to the corresponding driver.  Once it gets all the keys, it will then fetch all ping buffer data, update the Ping-Pong buffer groups (if both ping and pong buffers are empty, do nothing; if the ping buffer is full and pong buffer is empty, clear ping_data_rdy; if both ping and pong buffers are full, copy the pong buffer data to the ping buffer and clear pong_data_rdy), and put back all keys and write a test vector to STIL pattern.

Figure 13. stil_info_transaction properties.

Figure 14. STIL_generator block diagram.

# III.   DFT TDR Abstraction

## Idea Overview

For ultra-large-scale SoC, usually there is a group of TDRs, which are either IEEE 1500 or IEEE 1149.1 compliance, being used to configure the DFT design of a tile or a large design block.  The TDR groups among different tiles are chained together using IEEE 1687 protocol.  Fig. 15 is an example of DFT TDR access network.

It is necessary to level up TDR access in RAL, so as to make it easy to migrate UVM tests developing from this UVM-based DFT verification environment among verification environments and tests from block to system levels.  By doing this test writers can focus on test sequences as such, rather than the complex operation of accessing every TDR hierarchically located in the network.

For non-UVM-based environment, the normal way is to define a base class according to its protocol (for example, to define an IEEE1500 TDR base class and an IEEE 1149 TDR base class) and wrap up a TDR access operation inside its extension.  When DFT access network changes, the wrapped-up access operation in each TDR class has to be updated accordingly.  Such work is usually time-consuming.  However, the method of modelling DFT TDR in UVM-based environment is rarely seen in literature to the author’s knowledge.

This paper presents a neat and easy maintenance way to abstract TDR in UVM-based environment, as shown in Fig. 16.

We can encode a TDR’s location information into its address as shown in Fig. 17 and model an equivalent TDR access network named as dft_tdr_network in dft_tdr_monitor as shown in Fig. 16.

In Fig. 1, the reg2bus direction is shown in red lines, where dft_tdr_trans_to_jtag_trans_sequence fetches dft_tdr_transactions, unpacks address, decodes SIB code to get TDR location information, and then generates jtag_transactions to jtag_sequencer [3].  For the bus2reg direction shown in blue lines, dft_tdr_network maintains network status using jtag_transactions from jtag_monitor.  When sib_nodes value hit SIB code in dft_tdr_block, dft_tdr_monitor writes a dft_tdr_transaction to dft_tdr_predictor.

In this way, the TDR class definition is very neat, and only needs to declare each bit field of it.  Fig. 19 is an example of a TDR class definition.  When TDR access network changes, we only need to update dft_tdr_network and dft_tdr_trans_to_jtag_trans_sequence, while all TDR class definitions do not need any update that can save a lot of test environment setup time.

Figure 15. DFT TDR access network example.

Figure 16. dft_tdr_layering block diagram.

Figure 17.  TDR address encode.

## DFT TDR Access Network Modelling

In DFT TDR access network, a SIB bit and a TDR bit can be modelled as shown in Fig. 18.

out_update () method is to model the active clock edge triggering the shift register bit during shift operation and value_update () method is to model the active clock edge triggering the update register bit during update operation.

dft_tdr_network uses sib_node and reg_node to construct an equivalent network as DUT.

And it only needs to model each 1500 client’s IR and a WDR (Wrapper Data Register) whose length is dynamic, which can calculate from jtag_transaction coming from jtag_monitor and current network chain length.  It need not actually model every TDR, because each time only a TDR can be configured in a 1500 client.

## DFT TDR Class Definition

DFT TDR class definition is similar to other function registers, which extend from uvm_reg class.  A bypass TDR that has only one bit field is defined in Fig. 19 as an example.

Figure 18. TDR access network element modelling.

Figure 19. DFT TDR definition example.

## dft_tdr_transaction Class and bus_reg_ext Class

bus_reg_ext class is used for sending golden value to jtag_driver when doing register read or write in RAL.  dft_tdr_adpter colons the extension information to the handle of extension in dft_tdr_transaction in bus2reg direction.

Fig. 20 and Fig. 21 show all properties of dft_tdr_transaction and bus_reg_ext class.

read_not_write indicates current register operation is a UVM_READ or UVM_WRITE kind.

address is the encoded TDR address.

If current register access kind is UVM_WRITE, dft_tdr_adapter shift write data to wr_data_q.  If current register access kind is UVM_READ, dft_tdr_adapter shift the default value of the register to wr_data_q.

extension is an object of bus_reg_ext class.  It is used to transfer the side information for TDO pad checking in RAL.

reg_length stores current register’s length.

In bus2reg direction, if current register access kind is UVM_WRITE, dft_tdr_adapter returns data in wr_data_q else returns data in rd_data_q.

# IV.   STIL Test Pattern Verification

In order to verify the content and behaviours of the generated STIL file, we can use STIL VerifyTM to generate a Verilog testbench and re-run simulation before delivering to ATE test engineers.  The STIL file is verified if the simulation passes in STIL VerifyTM generated Verilog testbench.

STIL VerifyTM is a free verification utility provided by Mentor Graphics for checking the conformity of STIL files, which ensures that STIL files are syntactically correct, and features a Verilog testbench that allows EDA (Electronic Design Automation) and ATE tool developers to run and display STIL content in any Verilog simulator taking STIL file and DUT as input [4].

# V.   Discussion

In Fig. 1, pad_agent is mostly a physical layer agent that only drives and samples pads directed by pad_rw_transactions, and has no knowledge about interface protocols, although it groups pads based on their interface protocols.  If needed, the user can implement an upper layer agent to convert protocol-related transactions to pad_info_transactions and pass them down to pad_agent.

For the sake of simplification, this paper focuses on describing how to build a verification environment that can convert UVM tests to test patterns for ATE test during simulation, and the common components such as coverage collectors and scoreboards are not shown, the user can easily implement them using sequence items coming from jtag_monitor, dft_tdr_monitor, and pad_monitor.

Fig. 22 is an example of building an upper layer agent that includes a scoreboard and a coverage collector using sequence items from pad_monitor, above the pad_agent.  Inside scan_agnet, scan related protocols are implemented in scan_trans_to_pad_rw_trans_sequence, which converts each scan_transaction to a serial of pad_rw_transactions.  scan_monitor collects pad_rw_transactions and converts them into scan_transactions.

| Figure 20. dft_tdr_transaction properties definition. | Figure 21. bus_reg_ext properties definition. |
| --- | --- |

Figure 22. An example of building upper layer agent above pad_agent

Because we enforce every pad drive and sample should be done by a driver except for reference clock pads and each driver passes STIL information to the STIL_generator whenever it drivers and samples a pad, the generated STIL pattern is functionally equivalent to its corresponding UVM test.  Coverage statistics, which is gathered from coverage collectors to rank a UVM test, is also used to rate the generated STIL pattern.

# VI.   Conclusion

This UVM-based DFT environment can be easily adopted in most projects for DFT function verification by overriding dft_env_configuration, grouping pads as shown in Section B, and defining related interfaces.

By modifying the call_stil_gen () method in each driver to transfer pad toggle and measurement information to the format of the other test language required, this environment could also generate other format patterns ATE needs, not just the STIL format.

Using this method, it saves usually a team’s work to translate DFT function tests to STIL patterns in a project, and more important, it avoids errors introduced in the manual translation process to save turnaround debug efforts.

The approach to lift TDR in RAL is also a general way and can be applied in most projects by modelling related dft_tdr_network and overriding dft_tdr_trans_to_jtag_trans_sequence.  Moreover, it makes it easy to migrate UVM tests developing from this UVM-based DFT verification environment among verification environments and tests from block to system levels.

This UVM-based DFT environment works well in an experiment project and the generated STIL test pattern files pass simulation using STIL VerifyTM, which indicates it could be applied in real projects.

The next step of work will be to use this environment in real projects and validate it in post-silicon debug.

# References

[1]	IEEE Standard Test Interface Language (STIL) for Digital Test Vector Data, Section 5. STIL orientation and capabilities tutorial (informative), p. 10.

[2]	S. Sutherland, and T. Fitzpatrick, “UVM rapid adoption: a practical subset of UVM,” DVCON 2015, p. 21.

[3]	Mentor Graphics, Online UVM Cookbook (http://verificationacademy.com/cookbook), pp. 302–307.

[4]	Mentor Graphics (https://www.mentor.com/products/silicon-yield/getting_started_stil).